USGS

Download Samples - USGS Water Data for the Nation

- Pulled 1/14/2026

- 01/01/1997 to 01/01/2026

- TSS only

- Saved CSV to Excel, made two tabs – one for original, other for slimmed down columns

- Deleted data

  - Provisional

  - Estimated

  - “Major QC issue”

  - “Minor QC issue” off by 10-30%

- Manually assigned TSS value as ½ of MDL for non-detects

- Changed column names to match AWQMS

NARS – National Rivers & Streams Assessment

NARS Data Download Tool | US EPA

- Pulled 1/14/2026

- Downloaded all water chemistry data for Oregon during survey years 2008-09, 2013-14, and 2018-19.

- Manually merged years for columns of interest

  - Tier I: TN, TP, TSS, Turbidity, Conductivity, pH, Chloride, Sulfate, Chlorophyll

  - Tier II: nitrate, nitrite, silica, ammonia as N, DOC, calcium, magnesium, potassium, sodium

- No MDL listed for many parameters (e.g., TSS). Often a mix of MDLs for same method. Confusing so didn’t alter non-detects unless value was blank (e.g., nitrite 2018-19). Changed values = bolded.

- Changed column names to match AWQMS

BLM – AIM

aim | BLM GBP Hub

COMIDs

- Saved external data sources as CSV and ran through Dan’s COMID batch processing tool. Took around 3 hrs to process USGS’ 19,000 sites.

- Imported CSVs into R and isolated distinct records (i.e., didn’t want to review multiple COMIDs at the same station). Exported as new CSV.

- Made template in ArcGIS for review: NHD medium, NHD high, aerial, labels.

- Made stations shapefiles from CSVs.

- Reviewed each station by clicking through in attribute table. Took ~5 seconds per site, just making sure COMIDs matched.

- Made COMID corrections in spreadsheets (green highlights). Used -99999 for high res sites.